# Edge-Driven Urban Resilience: A Comprehensive Architecture and Multi-Sensor Fusion Approach for Real-Time Drainage Monitoring

Under The Guidance Of Dr. Ravi Babu Devareddi,GITAM

# 1st Jesica Michel

Dept. of Computer Science & Engineering GITAM University Hyderabad, India

jmichel@gitam.in

# 3rd Yamini Manda

Dept. of Computer Science & Engineering GITAM University Hyderabad, India

ymanda@gitam.in

# 2nd Hanvith Reddy Bhimireddy

Dept. of Computer Science & Engineering GITAM University Hyderabad, India

hbhmire@gitam.in

# 4th Tanish Das

Dept. of Computer Science & Engineering GITAM University Hyderabad, India

tdas2@gitam.in

Abstract—The rapid urbanization of smart cities has neces- sitated the deployment of massive Internet of Things (IoT) networks to monitor critical infrastructure. However, traditional centralized cloud computing architectures struggle to meet the low-latency and bandwidth-efficiency requirements of real-time disaster management systems. This paper presents a rigorous analysis of the Edge Computing paradigm, establishing a hi- erarchical taxonomy of edge devices and a five-layer reference architecture. We validate this architecture through the design and implementation of a Smart Drainage Pipeline Condition Monitoring System. Unlike existing solutions that rely on simple water-level thresholds, our system employs a multi-sensor fu- sion approach (Ultrasonic, Flow, Turbidity, Vibration) processed locally on an ESP32 edge node. We introduce four novel edge- resident algorithms—Sand Level Detection (SLDA), Pipe Water Volume (PWVA), Blockage Detection (BDA), and Dynamic Pipe Condition Classification (DPCC)—to detect sediment buildup and predict overflow events in real-time. Experimental results demonstrate that shifting intelligence to the edge reduces data transmission by over 97% and achieves blockage detection latency of under 40ms, proving the efficacy of edge computing for mission-critical urban applications.

Index Terms—Edge Computing, IoT, Smart City, Multi-Sensor Fusion, Drainage Monitoring, ESP32, Real-Time Systems.

[Image of System Architecture Diagram]

- Introduction

The proliferation of Internet of Things (IoT) devices is generating an unprecedented volume of data, projected to reach 175 zettabytes by 2025 [1]. In the context of smart cities, this data is critical for managing urban infrastructure such as traffic, waste management, and drainage systems. However, the traditional cloud-centric model, where all raw

data is transmitted to a centralized server for processing, faces severe limitations [2].

Firstly, latency becomes a critical bottleneck. In emergency scenarios, such as flash floods or rapid pipeline bursts, the round-trip time (RTT) required to send sensor data to the cloud, process it, and receive a command is often too high. Studies indicate that cloud-based systems can introduce delays ranging from 500ms to several seconds depending on network congestion, which is unacceptable for real-time disaster miti- gation [13].

Secondly, bandwidth consumption is a major concern. Continuous streaming of high-frequency sensor data (e.g., vibration or acoustic feeds) overwhelms network capacity and incurs high operational costs. A single drainage node streaming raw data at 1Hz can consume gigabytes of data monthly, making city-wide scaling economically unviable.

Finally, reliability is compromised in cloud-only models; if network connectivity fails during a storm, the monitoring system becomes useless [3]. This ”single point of failure” model is risky for critical infrastructure.

Edge Computing addresses these challenges by decentraliz- ing computation, moving data processing and storage closer to the data source. This paper explores the theoretical foundations of Edge Computing and applies them to a novel domain: Urban Drainage Monitoring.

- Literature Review

The domain of smart drainage monitoring has evolved from manual inspections to automated IoT solutions. However,

most existing work focuses on reactive alerting rather than predictive maintenance.

- Cloud-Based Manhole Monitoring

Early implementations, such as those by K. P. Arjun et al. [4], utilized Zigbee sensors to monitor gas levels and water height in manholes. While effective for worker safety, these systems suffered from high latency due to full cloud dependence. Recent studies by Yadav et al. [6] emphasize that centralized architectures struggle with real-time anomaly detection in large-scale urban networks due to bandwidth constraints.

- Blockage Detection Techniques

Current blockage detection primarily relies on simple level thresholds. Bhosale et al. [7] proposed a decision-tree ap- proach for stormwater drains but lacked internal volumetric analysis. Our work improves upon this by integrating a Multi- Sensor Fusion approach, a technique validated by Jaiswal et al. [8] as superior for fault diagnosis in industrial pipes.

- Energy Efficiency in IoT

Power consumption remains a critical challenge for un- derground sensors. Research by Broell et al. [9] on ESP32 optimization demonstrates that utilizing Deep Sleep modes can extend battery life by up to 500%. Our system adopts these findings, achieving a theoretical runtime of over 4 months (see Section X).

- Research Gap

Existing commercial solutions often require high-cost pro- prietary hardware. Our proposed solution bridges this gap by offering a low-cost (≈$33), edge-intelligent alternative suitable for high-density deployment in developing cities [15].

- Theoretical Background & Taxonomy

- Defining Edge Computing

Edge computing is defined as a distributed computing paradigm that brings computation and data storage closer to the location where it is needed, to improve response times and save bandwidth. It differs fundamentally from Cloud Computing in its topology: while the cloud relies on massive, centralized data centers, the edge utilizes a mesh of distributed nodes.

- Taxonomy of Edge Devices

A robust edge ecosystem comprises various device classes, which we classify as follows:

  - IoT Devices & Sensors (The ”Mist”): These are data- generating units with minimal compute power. In our system, this includes the HC-SR04 Ultrasonic Sensors and YF-S201 Flow Sensors.

  - Edge Gateways: These devices act as communica- tion hubs, translating protocols (e.g., Modbus/Zigbee to MQTT) and performing basic filtering. They bridge the gap between operational technology (OT) and informa- tion technology (IT).

  - Edge Nodes (Local Servers): High-performance em- bedded systems capable of running complex logic. In this study, the ESP32 microcontroller serves as the primary edge node, executing classification algorithms locally.

  - Smart End Devices: Autonomous units like drones or robots that can operate independently of the network.

- System Architecture

We propose a five-layer architecture designed to optimize the flow of information from the physical pipe environment to the cloud dashboard.

Layer 1: Perception Layer (Sensors) – This layer consists of the physical hardware deployed inside the pipeline, in- cluding Dual Ultrasonic Sensors for sand/water differentiation, Flow Sensors, and Vibration sensors.

Layer 2: Edge Processing Layer (ESP32) – The core innovation of our system lies here. The ESP32 collects raw signals and applies Exponential Moving Average (EMA) filtering to remove noise. It then executes the DPCC algorithm locally.

Layer 3: Communication Layer – Processed data and alerts are transmitted via MQTT, a lightweight publish/sub- scribe protocol ideal for unstable network environments.

Layer 4: Cloud Layer – The cloud serves as the repository for historical data. It runs long-term analytics to identify seasonal trends.

Layer 5: Application Layer – A web-based dashboard (e.g., ThingsBoard or Node-RED) visualizes the data for municipal engineers.

- Methodology: Algorithmic Framework

The system’s intelligence is derived from four custom algorithms running on the Edge Node.

- Data Pre-processing (Noise Reduction)

Drainage environments are noisy. To mitigate false spikes, we implemented an Exponential Moving Average (EMA) filter on the Edge Node. The filtered value St at time t is calculated as:

St = α · Yt + (1 − α) · St−1	(1)

Where Yt is the raw sensor reading and α is the smoothing factor (set to 0.2).

- Sand Level Detection Algorithm (SLDA)

SLDA solves the issue of distinguishing water from sand using dual sensors. Let D be the known diameter of the pipe. Let W be the water height measured by the top sensor. Let E be the empty space distance to the sand surface measured by the bottom sensor. The sand accumulation level (S) is calculated as:

S = D − (W + E)	(2)

If S exceeds a predefined threshold (e.g., 15% of D), a ”Sand Buildup” alert is triggered.

Fig. 1. End-to-End Data Flow Diagram illustrating the sensor fusion pipeline. Raw signals are processed locally on the ESP32 Edge Node using the SLDA and BDA algorithms.

- Pipe Water Volume Algorithm (PWVA)

To predict overflow, we must calculate volume, not just level. The PWVA treats the pipe as a cylinder.

V olume = π × r2 × h	(3)

Where r is the pipe radius and h is the current water level (W ). If V olume > 90% of total capacity, the system flags a ”High Flood Risk” state.

- Blockage Detection Algorithm (BDA)

This algorithm utilizes multi-sensor fusion. It monitors the correlation between Level (L), Flow (F ), and Vibration (V ).

- Dynamic Pipe Condition Classification (DPCC)

The DPCC acts as the final decision engine, classifying the pipe into one of six states.

- Implementation & Hardware Design

The prototype was built using an ESP32 microcontroller due to its dual-core architecture and built-in Wi-Fi/Bluetooth capabilities.

Algorithm 1 Blockage Detection Logic (Edge-Resident) Require: WaterLevel (W ), FlowRate (Q), Vibration (V ) Ensure: PipeStatus

1: ExpectedQ ← 2.5 × (W 1.5) {Regression Model}

2:  FlowIndex ← Q/ExpectedQ

3: if FlowIndex < 0.4 and W > NormalLevel then

4:	if V > V IBTHRESH then

5:	return ”PARTIALBLOCKTURBULENCE”

6:	else

7:	return ”FULLBLOCKSTAGNATION”

8:	end if

9: else if Q > MAXFLOW then

10:	return ”STORMSURGE”

11: else

12:	return ”NORMALFLOW”

13: end if

Fig. 2. Logic Flowchart for the Blockage Detection Algorithm (BDA).

Algorithm 2 DPCC State Machine Logic	 Require: SandLevel (S), BlockStatus (B), Volume (V ) Ensure: SystemState

1: if S > THRESHSAND then

2:	State ← SANDACCUMULATION

3: else if V > 90% then

4:	State ← OV ERFLOW RISK

5: else if B == PARTIAL then

6:	State ← PARTIALBLOCK

7: else if B == FULL then

8:	State ← FULLBLOCK

9: else

10:	State ← CLEAR

11: end if

12: return State

1  {

2	"id": "Node01",

3	"water_cm": 18.4,

4	"sand_cm": 2.3,

5	"flow_lpm": 12.7,

  - "state": "CLEAR",

  - "battery": 87

8  }

## 2. Partial Blockage Event Payload:

1  {

2	"id": "Node01",

3	"water_cm": 28.6,

4	"sand_cm": 2.4,

5	"flow_lpm": 4.1,

- "state": "PARTIAL_BLOCK",

- "battery": 82

8  }

- Database Schema

Data is stored in a time-series database (InfluxDB) to enable historical analysis. Key fields are detailed in Table II.

Fig. 3. State Transition Diagram for the Dynamic Pipe Condition Classifica- tion (DPCC) system. The ESP32 acts as a Finite State Machine.

A. Hardware Specifics & Pin Mapping

To ensure reproducibility and facilitate peer review, Table I details the exact GPIO configuration used in our prototype circuit.

TABLE II

Cloud Database Schema

| Field | Data Type |
| --- | --- |
| timestamp | datetime |
| node id | string |
| water level cm | float |
| sand level cm | float |
| flow lpm | float |
| state | string |

TABLE I

ESP32 Pin Mapping for Sensor Interface

| Sensor Module | Signal Type | ESP32 GPIO |
| --- | --- | --- |
| HC-SR04 (Top) | Trigger / Echo | GPIO 5 / 18 |
| JSN-SR04T (Bottom) | Trigger / Echo | GPIO 17 / 16 |
| YF-S201 Flow | Pulse Interrupt | GPIO 34 |
| Turbidity | Analog ADC | GPIO 35 |
| Vibration (SW-420) | Digital Input | GPIO 27 |
| Battery Monitor | Voltage Divider | GPIO 32 |
| OLED Display | I2C SDA / SCL | GPIO 21 / 22 |

- Network & Cloud Configuration

A professional-grade topic structure and payload format were designed to ensure scalability for city-wide deployment [12].

- MQTT Topic Structure

The system organizes data hierarchically:

  - city/hyderabad/drainage/node01/status

  - city/hyderabad/drainage/node01/alert

- Data Payload

Data is transmitted in JSON format. Below are actual payloads captured during testing.

## 1. Normal Operation Payload:

- Experimental Results and Analysis

- Sensor Calibration & Accuracy

To ensure technical soundness, all sensors were calibrated against known standards [10].

  - Ultrasonic Sensor (HC-SR04) Calibration: The sensor was tested against a standard steel ruler at 5cm intervals. As shown in Table III, the HC-SR04 showed an average error of

0.73 cm, which is consistent with the manufacturer’s tolerance of ±1cm.

TABLE III

Ultrasonic Sensor Calibration Data

| Actual (cm) | Measured (cm) | Error (cm) |
| --- | --- | --- |
| 5 | 5.2 | +0.2 |
| 10 | 10.3 | +0.3 |
| 20 | 20.6 | +0.6 |
| 30 | 30.7 | +0.7 |
| 40 | 40.8 | +0.8 |
| 50 | 50.9 | +0.9 |
| 60 | 60.8 | +0.8 |
| 70 | 71.0 | +1.0 |
| 80 | 80.7 | +0.7 |
| 90 | 91.1 | +1.1 |
| 100 | 101.2 | +1.2 |

  - Flow Sensor (YF-S201) Calibration: The sensor was calibrated by filling a container with a known volume of water and recording the pulse count.

TABLE IV

Flow Sensor Calibration Data

| Liters Filled | Pulse Count |
| --- | --- |
| 0.5 | 221 |
| 1.0 | 448 |
| 1.5 | 676 |
| 2.0 | 903 |
| 3.0 | 1354 |
| 4.0 | 1806 |
| 5.0 | 2258 |

Linear regression of the data in Table IV yielded the following calibration equation:

|  | Pred: Clear | Pred: Partial | Pred: Full | Accuracy |
| --- | --- | --- | --- | --- |
| Act: Clear | 48 | 2 | 0 | 96% |
| Act: Partial | 3 | 45 | 2 | 90% |
| Act: Full | 0 | 1 | 49 | 98% |

Pulses

TABLE V

Defined Test Scenarios for System Validation

| Case ID | Scenario Description | Algorithm |
| --- | --- | --- |
| TC-01 | Clear Flow: Clean water pumped at variable rates (5-20 L/min). | Monitor |
| TC-02 | Sediment Buildup: 2cm layer of sand added to pipe bottom. | SLDA |
| TC-03 | Partial Block: Obstruction placed at pipe outlet; water rises, flow drops. | BDA |
| TC-04 | Flash Flood: Rapid water injection (>30 L/min) exceeding capacity. | PWVA |

TABLE VI

Confusion Matrix for Pipe Condition Classification

Liters =

451.6

(4)

This indicates a calibration factor of 451.6 pulses per liter, matching closely with the YF-S201 datasheet specification of

≈ 450 pulses per liter [11].

- Network Bandwidth Analysis

A key hypothesis was that Edge Computing saves band- width. We mathematically validated this.

- Raw Streaming (Cloud Model): Sending 4 floats (16 bytes) at 1Hz = 57.6 KB/hour.

- Edge Processing (Our Model): Sending 1 status packet (100 bytes) every 5 mins = 1.2 KB/hour.

## Result:

- Latency Comparison

We measured the ”Time-to-Alert” (∆t). The Edge-based system achieved a latency of 40 ms, significantly faster than the 1350 ms observed in cloud-based tests.

Savings =

57.6 − 1.2

57.6

× 100 ≈ 97.9%	(5)

This drastic reduction proves the system’s viability for cellular (GPRS) deployment.

- Algorithmic Time Complexity

The computational efficiency of the edge algorithms was an- alyzed to ensure real-time performance on the limited ESP32 hardware.

- SLDA: Basic arithmetic operations. Time Complexity:

O(1).

- BDA: Conditional checks on 3 variables. Time Complex- ity: O(1).

- DPCC: Finite State Machine with fixed states. Time Complexity: O(1).

The constant time complexity ensures that decision latency remains < 40ms regardless of data volume.

- System Validation Test Cases

We subjected the system to four distinct stress tests (Table V).

- Classification Accuracy

The system was tested with 50 trials for each scenario. Table VI illustrates the classification performance of the DPCC algorithm.

The system achieved an overall accuracy of 94.6%.

Fig. 4. Q-H Curve Analysis. The solid line represents the theoretical flow rate (Manning’s Eq). Points falling significantly below the curve indicate a Flow Deficit, triggering a Blockage Alert.

- Discussion: Competitive Analysis

To demonstrate novelty, we compared our solution against standard industry alternatives [?].

- Societal and Environmental Impact

The deployment of this system aligns directly with UN Sustainable Development Goals (SDGs), specifically SDG 11 (Sustainable Cities) and SDG 3 (Good Health), by preventing urban flooding and reducing vector-borne diseases caused by stagnant water [14].

TABLE VII

Comparative Analysis of Drainage Monitoring Solutions

| Feature | Our Solution | SCADA Systems | LoRaWAN Nodes |
| --- | --- | --- | --- |
| Primary Tech | Edge AI + Wi- Fi | PLC + Wired | Low-Power RF |
| Cost / Node | ≈$33 USD | > $5, 000 USD | ≈$150 USD |
| Real-Time? | Yes (40ms) | Yes (1-2s) | No (5-10 min) |
| Blockage Detection | Yes (Predictive) | No (Reactive) | No (Threshold) |
| Maintenance | Low (Modular) | High (Special- ized) | Low |

- Power Consumption Analysis

Energy efficiency is critical for off-grid IoT. We measured the current consumption of the ESP32 across various opera- tional states using a high-precision multimeter. The results are detailed in Table VIII.

TABLE VIII

Measured Power Consumption Profile

| Operational Mode | Current (mA) |
| --- | --- |
| Boot / Initialization | 120 mA |
| Wi-Fi Connecting | 170 – 210 mA |
| Wi-Fi Connected (Idle) | 70 – 90 mA |
| MQTT Publish Burst | 160 – 180 mA |
| Sensor Reading Only | 45 – 55 mA |
| Light Sleep | 8 – 12 mA |
| Deep Sleep | 0.014 mA |

During transmission bursts, the ESP32 consumed a peak of 182 mA. However, by leveraging Deep Sleep mode (0.014 mA), the average power consumption is drastically reduced, enabling months of operation on standard batteries.

- Challenges and Limitations

Real-world deployment faces hurdles such as sensor fouling (mud coating sensors) and signal attenuation in underground pipes. We propose self-cleaning mechanisms and external LoRaWAN antennas for future iterations.

- Security and Privacy

Security is enforced via MQTT Client ID Authentication and TLS/SSL encryption. Data integrity is ensured using HMAC signatures on all payloads to prevent injection attacks.

- Cost Analysis

A key objective was to create a low-cost alternative to industrial SCADA systems. Table IX presents the Bill of Materials (BOM) based on current market prices.

At approximately $33 USD per node, our solution is orders of magnitude cheaper than commercial equivalents, enabling high-density deployment.

TABLE IX

Bill of Materials (Prototype Cost in INR)

| Component | Price (INR) |
| --- | --- |
| ESP32 Dev Board | 380 |
| HC-SR04 (Top Sensor) | 75 |
| JSN-SR04T (Waterproof) | 220 |
| YF-S201 Flow Sensor | 260 |
| Turbidity Sensor | 190 |
| Vibration Sensor (SW-420) | 80 |
| Li-ion Battery (10Ah) | 850 |
| MT3608 Boost Converter | 45 |
| Jumper Wires + PCB | 100 |
| Enclosure & PVC Mounts | 420 |
| Misc (glue, tape, screws) | 100 |
| Total Cost | 2,720 (≈$33 USD) |

- Conclusion

This paper validated the use of Edge Computing in urban drainage monitoring. Through rigorous experimental testing and calibration, we demonstrated that our multi-sensor fusion approach, running on low-cost hardware, achieves high accu- racy and ultra-low latency. The integration of SLDA and BDA algorithms allows the detection of sediment buildup and partial blockages that existing systems miss.

References

- W. Shi, J. Cao, Q. Zhang, Y. Li, and L. Xu, ”Edge Computing: Vision and Challenges,” IEEE Internet of Things Journal, vol. 3, no. 5, pp. 637-646, Oct. 2016.

- M. Satyanarayanan, ”The Emergence of Edge Computing,” IEEE Com- puter, vol. 50, no. 1, pp. 30-39, Jan. 2017.

- ”IoT-Enabled Smart Manhole Management System,” ResearchGate, 2025.

- ”IoT Based smart drainage monitoring and cleaning system,” ScienceDi- rect, 2020.

- ”Real Time Drainage Monitoring System,” ResearchGate, 2025.

- H. L. Yadav et al., ”Integrating Edge Computing and IoT for Real-Time Air and Water Quality Monitoring Systems,” International Journal of Environmental Sciences, July 2025.

- P. Bhosale et al., ”IoT-Based Detection of Blockages in Stormwater Drains using Decision Trees,” MDPI Engineering Proceedings, vol. 82, no. 1, 2024.

- A. Jaiswal and A. Dutta, ”A Comprehensive Review On Deep Learning- Based Data Fusion for Industrial Fault Diagnosis,” IEEE Access, vol. 12,

pp. 1500-1520, 2025.

- L. M. Broell, C. Hanshans, and D. Kimmerle, ”IoT on an ESP32: Optimization Methods Regarding Battery Life and Deep Sleep Modes,” ResearchGate, 2024.

- D. Biswas and S. Roy, ”IoT-enabled smart water quality monitoring system: A case study on sensor calibration,” Environmental Monitoring and Assessment, vol. 194, no. 3, 2022.

- Espressif Systems, ”ESP32 Series Datasheet,” 2023.

- OASIS, ”MQTT Version 3.1.1,” OASIS Standard, 2014.

- Cisco, ”Cisco Global Cloud Index: Forecast and Methodology, 2016–2021,” White Paper, 2018.

- ”Sustainable Development Goal 11,” United Nations, [Online]. Avail- able: https://sdgs.un.org/goals/goal11.

- V. Gerlin et al., ”Smart Sewage System with Advanced Remote Moni- toring for Efficient Urban Infrastructure,” IEEE Xplore, March 2024.